*----------------------------------------------------------------------------------------------------------------------*

* load sets and parameters from dataset gdx                                                                            *

*----------------------------------------------------------------------------------------------------------------------*

put_utility 'log' /"+++ Importing data from '%in%'... +++ " ;

* all sets and general parameters from the gdx file

$GDXIN '%in%'

$LOAD node, tec=technology, year_all=year, commodity, level, grade, mode, time, rating

$LOAD emission, land_scenario, land_type, relation

$LOAD level_resource, level_renewable

$LOAD lvl_spatial, lvl_temporal, map_spatial_hierarchy, map_temporal_hierarchy

$LOAD map_node, map_time, map_commodity, map_resource, map_stocks, map_tec, map_tec_time, map_tec_mode

$LOAD is_capacity_factor

$LOAD map_land, map_relation

$LOAD type_tec, cat_tec, type_year, cat_year, type_emission, cat_emission, type_tec_land

$LOAD inv_tec, renewable_tec

$LOAD balance_equality, time_relative

$LOAD shares

$LOAD addon, type_addon, cat_addon, map_tec_addon

$LOAD storage_tec, level_storage, map_tec_storage

* Version information; conditional load to allow older GDX files

$ifthen gdxSetType ixmp_version

$load ixmp_version

$endif

$GDXIN

* Load indexed sets and parameters

Execute_load '%in%',

abs_cost_activity_soft_lo,

abs_cost_activity_soft_up,

abs_cost_new_capacity_soft_lo,

abs_cost_new_capacity_soft_up,

addon_conversion,

addon_lo,

addon_up,

bound_activity_lo,

bound_activity_up,

bound_emission,

bound_extraction_up,

bound_new_capacity_lo,

bound_new_capacity_up,

bound_total_capacity_lo,

bound_total_capacity_up,

capacity_factor,

commodity_stock,

construction_time,

demand_fixed = demand,

duration_period,

duration_time,

dynamic_land_lo,

dynamic_land_up,

emission_factor,

emission_scaling,

fix_cost,

fixed_activity,

fixed_capacity,

fixed_extraction,

fixed_land,

fixed_new_capacity,

fixed_stock,

flexibility_factor,

growth_activity_lo,

growth_activity_up,

growth_land_lo,

growth_land_scen_lo,

growth_land_scen_up,

growth_land_up,

growth_new_capacity_lo,

growth_new_capacity_up,

historical_activity,

historical_emission,

historical_extraction,

historical_land,

historical_new_capacity,

initial_activity_lo,

initial_activity_up,

initial_land_lo,

initial_land_scen_lo,

initial_land_scen_up,

initial_land_up,

initial_new_capacity_lo,

initial_new_capacity_up,

input,

interestrate,

inv_cost,

is_bound_activity_up,

is_bound_emission,

is_bound_extraction_up,

is_bound_new_capacity_lo,

is_bound_new_capacity_up,

is_bound_total_capacity_lo,

is_bound_total_capacity_up,

is_dynamic_activity_lo,

is_dynamic_activity_up,

is_dynamic_land_lo,

is_dynamic_land_scen_lo,

is_dynamic_land_scen_up,

is_dynamic_land_up,

is_dynamic_new_capacity_lo,

is_dynamic_new_capacity_up,

is_fixed_activity,

is_fixed_capacity,

is_fixed_extraction,

is_fixed_land,

is_fixed_new_capacity,

is_fixed_stock,

is_relation_lower,

is_relation_upper,

land_cost,

land_emission,

land_input,

land_output,

land_use,

level_cost_activity_soft_lo,

level_cost_activity_soft_up,

level_cost_new_capacity_soft_lo,

level_cost_new_capacity_soft_up,

map_shares_commodity_share,

map_shares_commodity_total,

min_utilization_factor,

operation_factor,

output,

peak_load_factor,

rating_bin,

relation_activity,

relation_cost,

relation_lower,

relation_new_capacity,

relation_total_capacity,

relation_upper,

reliability_factor,

renewable_capacity_factor,

renewable_potential,

resource_cost,

resource_remaining,

resource_volume,

share_commodity_lo,

share_commodity_up,

share_mode_lo,

share_mode_up,

soft_activity_lo,

soft_activity_up,

soft_new_capacity_lo,

soft_new_capacity_up,

storage_initial,

storage_self_discharge,

tax_emission,

technical_lifetime,

time_order,

var_cost

;

$IFTHEN %MESSAGE_CAP_COMM% == "1"

execute_load '%in%',

input_cap,

input_cap_new,

input_cap_ret,

output_cap,

output_cap_new,

output_cap_ret

;

$ENDIF

*----------------------------------------------------------------------------------------------------------------------*

* Sets derived from input sets                                                                                         *

*----------------------------------------------------------------------------------------------------------------------*

* 'yes' for any type_tec that is a member of map_shares_commodity_share with any combination of other indices

type_tec_share(type_tec)$SUM(

(shares, ns, n, m, commodity, l),

map_shares_commodity_share(shares, ns, n, type_tec, m, commodity, l)

) = YES;

* Same, except with map_shares_commodity_total

type_tec_total(type_tec)$SUM(

(shares, ns, n, m, commodity, l),

map_shares_commodity_total(shares, ns, n, type_tec, m, commodity, l)

) = YES;

*----------------------------------------------------------------------------------------------------------------------*

* ensure that each node is mapped to itself                                                                            *

*----------------------------------------------------------------------------------------------------------------------*

map_node(node,node) = yes ;

*----------------------------------------------------------------------------------------------------------------------*

* ensure that map_tec_extended includes entire model horizon when lifetime is defined.

*----------------------------------------------------------------------------------------------------------------------*

$IFTHEN %MESSAGE_CAP_COMM% == "1"

map_tec_extended(node,tec,year_all)$technical_lifetime(node,tec,year_all) = yes;

$ENDIF

*----------------------------------------------------------------------------------------------------------------------*

* auxiliary mappings for the implementation of bounds over all modes and system reliability/flexibility constraints    *

*----------------------------------------------------------------------------------------------------------------------*

Set all_modes (mode) ;

all_modes('all') = yes ;

Set rating_unfirm(rating) ;

rating_unfirm(rating) = yes ;

rating_unfirm('firm') = no ;

Set rating_unrated(rating) ;

rating_unrated(rating) = yes ;

rating_unrated('unrated') = no ;

*----------------------------------------------------------------------------------------------------------------------*

* assignment and computation of MESSAGE-specific auxiliary parameters                                                  *

*----------------------------------------------------------------------------------------------------------------------*

* get assignment of auxiliary parameter for period mappings and duration

$INCLUDE includes/period_parameter_assignment.gms

* compute auxiliary parameters for relative duration of subannual time periods

duration_time_rel(time,time2)$( map_time(time,time2) ) = duration_time(time2) / duration_time(time) ;

* making duration_time_rel equal to 1, i.e., a consistent unit for ACT in sub-annual time slices, for parent 'time' not specified in set 'time_relative'

duration_time_rel(time,time2)$( Not time_relative(time) ) = 1 ;

* assign an additional mapping set for technologies to nodes, modes and subannual time slices (for shorter reference)

map_tec_act(node,tec,year_all,mode,time)$( map_tec_time(node,tec,year_all,time) AND

map_tec_mode(node,tec,year_all,mode) ) = yes ;

* mapping of technology lifetime to all 'current' periods (for all non-investment technologies)

map_tec_lifetime(node,tec,year_all,year_all)$map_tec(node,tec,year_all) = yes;

* mapping of technology lifetime to all periods 'year_all' which are within the economic lifetime

* (if built in period 'vintage')

map_tec_lifetime(node,tec,vintage,year_all)$(

map_tec(node,tec,vintage)

AND map_tec(node,tec,year_all)

AND map_period(vintage,year_all)

AND duration_period_sum(vintage,year_all) < technical_lifetime(node,tec,vintage)

) = yes;

* mapping of technology lifetime to all periods 'year_all' which were built prior to the beginning of the model horizon

map_tec_lifetime(node,tec,historical,year_all)$(

map_tec(node,tec,year_all)

AND map_period(historical,year_all)

AND historical_new_capacity(node,tec,historical)

AND duration_period_sum(historical,year_all) < technical_lifetime(node,tec,historical)

) = yes;

* Same conditions as map_tec_lifetime, except inclusive of historic periods via map_tec_extended.

* If MESSAGE_CAP_COMM is "0", map_tec_extended is empty and all these are no-ops.

map_tec_lifetime_extended(node,tec,year_all,year_all)$map_tec_extended(node,tec,year_all) = yes;

map_tec_lifetime_extended(node,tec,vintage,year_all)$(

map_tec_extended(node,tec,vintage)

AND map_tec_extended(node,tec,year_all)

AND map_period(vintage,year_all)

AND duration_period_sum(vintage,year_all) < technical_lifetime(node,tec,vintage)

) = yes;

map_tec_lifetime_extended(node,tec,historical,year_all)$(

map_tec_extended(node,tec,year_all)

AND map_period(historical,year_all)

AND historical_new_capacity(node,tec,historical)

AND duration_period_sum(historical,year_all) < technical_lifetime(node,tec,historical)

) = yes;

* NB map_cap_ret_hist_* are populated after remaining_capacity_extended in scaling_investment_costs.gms

map_cap_ret(node,tec,vintage,y_prev,y_all)$(

inv_tec(tec)

AND map_tec_lifetime(node,tec,vintage,y_prev)

AND NOT first_period(y_all)

AND seq_period(y_prev,y_all)

) = yes;

* mapping of renewable technologies to their input commodities

map_ren_com(node,renewable_tec,commodity,year_all)$(

SUM((node2,year_all2,mode,level_renewable,time_act,time),

input(node2,renewable_tec,year_all,year_all2,mode,node,commodity,level_renewable,time_act,time) ) ) = yes;

* mapping of renewable commodities to grades

map_ren_grade(node,commodity,grade,year_all)$(

SUM(level_renewable, renewable_potential(node,commodity,grade,level_renewable,year_all) ) ) = yes;

* mapping of technologies to commodities and ratings

map_rating(node,inv_tec,commodity,level,rating,year_all)$(

SUM(time, reliability_factor(node,inv_tec,year_all,commodity,level,time,rating) ) ) = yes;

* assign the yearly average capacity factor (used in equation OPERATION_CONSTRAINT)

capacity_factor(node,tec,year_all2,year_all,'year') =

sum(time$map_tec_time(node,tec,year_all,time), duration_time(time)

* capacity_factor(node,tec,year_all2,year_all,time) );

* update the masking set for capacity factor based on the average values added above

is_capacity_factor(node,tec,year_all2,year_all,time)$(capacity_factor(node,tec,year_all2,year_all,'year') ) = yes;

* set the default capacity factor for technologies where no parameter value is provided in the input data

capacity_factor(node,tec,year_all2,year_all,time)$( map_tec_time(node,tec,year_all,time)

AND map_tec_lifetime(node,tec,year_all2,year_all) AND NOT is_capacity_factor(node,tec,year_all2,year_all,time) ) = 1 ;

* set the default operation factor for technologies where no parameter value is provided in the input data

operation_factor(node,tec,year_all2,year_all)$( map_tec(node,tec,year_all)

AND map_tec_lifetime(node,tec,year_all2,year_all) AND NOT operation_factor(node,tec,year_all2,year_all) ) = 1 ;

* set the upper bound on addon-technology activity to 1 by default

addon_up(node,tec,year_all,mode,time,type_addon)$(

map_tec_addon(tec,type_addon)

AND map_tec_act(node,tec,year_all,mode,time)

AND NOT addon_up(node,tec,year_all,mode,time,type_addon) ) = 1 ;

* set the emission scaling parameter to 1 by default

emission_scaling(type_emission,emission)$( cat_emission(type_emission,emission)

and not emission_scaling(type_emission,emission) ) = 1 ;

* mapping of storage technologies to their level and commodity (can be different from level and commodity of storage media)

map_time_commodity_storage(node,tec,level,commodity,mode,year_all,time)$( storage_tec(tec) AND

SUM( (node2,year_all2,time_act), input(node2,tec,year_all,year_all2,mode,node,commodity,level,time_act,time) ) ) = yes;

* mapping of sequence of sub-annual time slices in a period and temporal level

map_time_period(year_all,lvl_temporal,time,time2)$( time_order(lvl_temporal,time) AND

time_order(lvl_temporal,time) + 1 = time_order(lvl_temporal,time2) ) = yes;

* mapping of sequence of the last sub-annual time slice to the first to create a close the order of time slices

map_time_period(year_all,lvl_temporal,time,time2)$( time_order(lvl_temporal,time) AND

time_order(lvl_temporal,time) = SMAX(time3,time_order(lvl_temporal,time3) ) AND time_order(lvl_temporal,time2) = 1 ) = yes;

*----------------------------------------------------------------------------------------------------------------------*

* sanity checks on the data set                                                                                        *

*----------------------------------------------------------------------------------------------------------------------*

Parameter check ;

* check whether all relevant technology/vintage/year combinations have positove input/output values assigned

*loop((node,tec,vintage,year_all)$( map_tec_lifetime(node,tec,vintage,year_all) ),

*    if ( sum( (mode,node2,commodity,level,time,time2),

*            input(node,tec,vintage,year_all,mode,node2,commodity,level,time,time2)

*            + output(node,tec,vintage,year_all,mode,node2,commodity,level,time,time2) ) eq 0 ,

*        put_utility 'log'/" Warning: No input or output not defined for '"node.tl:0"|"tec.tl:0"|"vintage.tl:0"|"year_all.tl:0"' !" ;

*    ) ;

*) ;

* check that the economic and technical lifetime are defined and consistent for all investment technologies

loop((node,inv_tec,model_horizon)$( map_tec(node,inv_tec,model_horizon) ),

if ( technical_lifetime(node,inv_tec,model_horizon) <= 0 ,

put_utility 'log'/" Error: Technical lifetime not defined for '"node.tl:0"|"inv_tec.tl:0"|"model_horizon.tl:0"' !" ;

check = 1 ;

) ;

) ;

if (check,

abort "There is a problem with the definition of the technical lifetime!" ;

) ;

* check for validity of temporal resolution with an accepted difference in the scale of 1e-12, due to the different percision between GAMS and python values.

loop(lvl_temporal,

loop(time2$( sum(time, map_temporal_hierarchy(lvl_temporal,time,time2) ) ),

check = 1$( abs( sum( time$( map_temporal_hierarchy(lvl_temporal,time,time2) ),

duration_time(time) ) - duration_time(time2) ) > 1e-12 );

) ;

) ;

if (check,

abort "There is a problem in the definition of the temporal resolution!" ;

);

* check that the resources-remaining parameter is in the interval (0,1]

loop( (node,commodity,grade,year_all)$( map_resource(node,commodity,grade,year_all)

AND resource_remaining(node,commodity,grade,year_all) ),

if( ( resource_remaining(node,commodity,grade,year_all) > 1

or resource_remaining(node,commodity,grade,year_all) <= 0 ),

put_utility 'log'/" Error: Inconsistent value of parameter 'resources_remaining' "

"for '"node.tl:0"|"commodity.tl:0"|"grade.tl:0"|"year_all.tl:0 "' !" ;

check = 1 ;

) ;

) ;

if (check,

abort "There is a problem with the parameter 'resources_remaining'!" ;

) ;

* check that the sum of rating bins (if used for firm-cacpacity or flexibility) is greater than 1

loop( (node,tec,year_all,commodity,level,time)$(

( sum((vintage,rating_unfirm), reliability_factor(node,tec,year_all,commodity,level,time,rating_unfirm) )

OR sum((vintage,mode,rating_unrated)$(

flexibility_factor(node,tec,vintage,year_all,mode,commodity,level,time,rating_unrated) ), 1 ) )

),

if ( ( sum( rating, rating_bin(node,tec,year_all,commodity,level,time,rating) ) < 1 ),

put_utility 'log'/" Error: Insufficient rating bin assignment ' "

"for '"node.tl:0"|"tec.tl:0"|"year_all.tl:0 "'" ;

check = 1 ;

) ;

) ;

if (check,

abort "There is a problem with assignment of rating bins!" ;

) ;