***

* Auxiliary investment parameters

* ===============================

*

***

* compute the scaling investment parameters for all periods in the model horizon

year(year_all)$( model_horizon(year_all) ) = yes ;

* compute the technical lifetime remaining beyond the overall model horizon

beyond_horizon_lifetime(node,inv_tec,vintage)$( map_tec(node,inv_tec,vintage) ) =

technical_lifetime(node,inv_tec,vintage) - remaining_years(vintage) ;

beyond_horizon_lifetime(node,inv_tec,vintage)$( beyond_horizon_lifetime(node,inv_tec,vintage) < 0 ) = 0 ;

***

* Levelized costs excluding fuel costs

* ------------------------------------

* For the 'soft' relaxations of the dynamic constraints and the associated penalty factor in the objective function,

* we need to compute the parameter :math:`\text{levelized_cost}_{n,t,y}`.

*

* .. math::

*    \text{levelized_cost}_{n,t,m,y,h} := \

*        & \text{inv_cost}_{n,t,y} \cdot \frac{ \text{interestrate}_{y} \cdot \left( 1 + \text{interestrate}_{y} \right)^{|y|} }

*                                      { \left( 1 + \text{interestrate}_{y} \right)^{|y|} - 1 } \\

*        & + \text{fix_cost}_{n,t,y,y} \cdot \frac{ 1 }{ \sum_{h'} \text{duration_time}_{h'} \cdot \text{capacity_factor}_{n,t,y,y,h'} } \\

*        & + \text{var_cost}_{n,t,y,y,m,h}

*

* where :math:`|y| = \text{technical_lifetime}_{n,t,y}`. This formulation implicitly assumes constant fixed

* and variable costs over time.

*

* .. warning:: Levelized capital costs do not include fuel-related costs.

*    All soft relaxations of the dynamic activity constraint are disabled if the levelized costs are negative.

***

levelized_cost(node,tec,year,time)$( map_tec_time(node,tec,year,time)) =

(inv_cost(node,tec,year)

* (

* compute discounted annualized investment costs if interest rate > 0

( interestrate(year)

* ( 1 + interestrate(year) ) ** technical_lifetime(node,tec,year)

/ ( ( 1 + interestrate(year) ) ** technical_lifetime(node,tec,year) - 1 )

)$( interestrate(year) )

* if interest rate = 0, annualized investment costs are total investment costs divided by technical lifetime

+ ( 1 / technical_lifetime(node,tec,year) )$( interestrate(year) eq 0 )

)

* add (proportional) fixed and variable costs, assuming that these remain constant over the technical lifetime

+ ( fix_cost(node,tec,year,year) /

sum(time2$( map_tec_time(node,tec,year,time2) ),

duration_time(time2) * capacity_factor(node,tec,year,year,time2) )

)$( fix_cost(node,tec,year,year) ))$(inv_tec(tec))

+ sum(mode$( map_tec_act(node,tec,year,mode,time) ), var_cost(node,tec,year,year,mode,time) )

;

* the soft relaxations of the dynamic activity constraints are disabled if the levelized costs are negative

loop((node,tec,year,time)$( levelized_cost(node,tec,year,time) < 0

AND ( soft_activity_up(node,tec,year,time) + soft_activity_lo(node,tec,year,time) ) > 0 ),

put_utility 'log' /'Remove relaxations for dynamic activity constraints for ',node.tl,'|',tec.tl,'|',year.tl,'!' ;

soft_activity_up(node,tec,year,time) = 0 ;

soft_activity_lo(node,tec,year,time) = 0 ;

) ;

***

* Construction time accounting

* ----------------------------

* If the construction of new capacity takes a significant amount of time,

* investment costs have to be scaled up accordingly to account for the higher capital costs.

*

* .. math::

*    \text{construction_time_factor}_{n,t,y} = \left( 1 + \text{interestrate}_y \right)^{|y|}

*

* where :math:`|y| = \text{construction_time}_{n,t,y}`. If no construction time is specified, the default value of the

* investment cost scaling factor defaults to 1. The model assumes that the construction time only plays a role

* for the investment costs, i.e., each unit of new-built capacity is available instantaneously.

*

* **Comment:** This formulation applies the discount rate of the vintage year

* (i.e., the year in which the new capacity becomes operational).

***

* set default construction_time_factor to 1

construction_time_factor(node,inv_tec,year)$( map_tec(node,inv_tec,year) ) = 1;

* compute the construction_time_factor

construction_time_factor(node,inv_tec,year)$( map_tec(node,inv_tec,year) AND construction_time(node,inv_tec,year) ) =

( 1 + interestrate(year) ) ** construction_time(node,inv_tec,year) ;

***

* Investment costs beyond the model horizon

* -----------------------------------------

* If the technical lifetime of a technology exceeds the model horizon :math:`Y^{\text{model}}`, the model has to add

* a scaling factor to the investment costs (:math:`\text{end_of_horizon_factor}_{n,t,y}`). Assuming a constant

* stream of revenue (marginal value of the capacity constraint), this can be computed by annualizing investment costs

* from the condition that in an optimal solution, the investment costs must equal the discounted future revenues,

* if the investment variable :math:`\text{CAP_NEW}_{n,t,y} > 0`:

*

* .. math::

*    \text{inv_cost}_{n,t,y^V} = \sum_{y \in Y^{\text{lifetime}}_{n,t,y^V}} \text{df_year}_{y} \cdot \beta_{n,t},

*

* Here, :math:`\beta_{n,t} > 0` is the dual variable to the capacity constraint (assumed constant over future periods)

* and :math:`Y^{\text{lifetime}}_{n,t,y^V}` is the set of periods in the lifetime of a plant built in period :math:`y^V`.

* Then, the scaling factor :math:`\text{end_of_horizon_factor}_{n,t,y^V}` can be derived as follows:

*

* .. math::

*    \text{end_of_horizon_factor}_{n,t,y^V} :=

*    \frac{\sum_{y \in Y^{\text{lifetime}}_{n,t,y^V} \cap Y^{\text{model}}} \text{df_year}_{y} }

*        {\sum_{y' \in Y^{\text{lifetime}}_{n,t,y^V}} \text{df_year}_{y'} + \text{beyond_horizon_factor}_{n,t,y^V} }

*    \in (0,1],

*

* where the parameter :math:`\text{beyond_horizon_factor}_{n,t,y^V}` accounts for the discount factor beyond the

* overall model horizon (the set :math:`Y` in contrast to the set :math:`Y^{\text{model}} \subseteq Y` of the periods

* included in the current model iteration (see the page on the recursive-dynamic model solution approach).

*

* .. math::

*    \text{beyond_horizon_lifetime}_{n,t,y^V} :=  \max \Big\{ 0,

*        \text{economic_lifetime}_{n,t,y^V} - \sum_{y' \geq y^V} \text{duration_period}_{y'} \Big\}

*

* .. math::

*    \text{beyond_horizon_factor}_{n,t,y^V} :=

*        \text{df_year}_{\widehat{y}} \cdot \frac{1}{ \left( 1 + \text{interestrate}_{\widehat{y}} \right)^{|\widehat{y}|} }

*        \cdot \frac{ 1 - \left( \frac{1}{1 + \text{interestrate}_{\widehat{y}}} \right)^{|\widetilde{y}|}}

*                   { 1 - \frac{1}{1 + \text{interestrate}_{\widehat{y}}}}

*

* where :math:`\widehat{y}` is the last period included in the overall model horizon,

* :math:`|\widehat{y}| = \text{duration_period}_{\widehat{y}}`

* and :math:`|\widetilde{y}| = \text{beyond_horizon_lifetime}_{n,t,y^V}`.

*

* If the interest rate is zero, i.e., :math:`\text{interestrate}_{\widehat{y}} = 0`,

* the parameter :math:`\text{beyond_horizon_factor}_{n,t,y^V}` equals the remaining technical lifetime

* beyond the model horizon and the parameter :math:`\text{end_of_horizon_factor}_{n,t,y^V}` equals

* the share of technical lifetime within the model horizon.

***

* compute the cumulative discount factor of the technical lifetime remaining beyond the model horizon

beyond_horizon_factor(node,inv_tec,vintage)$( beyond_horizon_lifetime(node,inv_tec,vintage) )

= sum(last_period,

* compute the discount factor of the very last year (not period) in the model horizon

df_year(last_period) * (

* multiply this by the geometric series of remaining technical lifetime if interestrate of last model period > 0

(

( 1 - POWER( 1 / ( 1 + interestrate(last_period) ), beyond_horizon_lifetime(node,inv_tec,vintage) ) )

/ ( 1 - 1 / ( 1 + interestrate(last_period) ) )

)$( interestrate(last_period) )

* if interest rate = 0, multiply by remaining technical lifetime

+ ( beyond_horizon_lifetime(node,inv_tec,vintage) )$( interestrate(last_period) eq 0 )

)

) ;

* deterine the parameter end_of_horizon_factor used for scaling investment costs to account for

* technical lifetime beyond the model horizon

end_of_horizon_factor(node,inv_tec,vintage)$( map_tec(node,inv_tec,vintage) ) =

sum(year_all$( map_tec_lifetime(node,inv_tec,vintage,year_all) ), df_period(year_all)  )

/ ( sum(year_all$( map_tec_lifetime(node,inv_tec,vintage,year_all) ), df_period(year_all) )

+ beyond_horizon_factor(node,inv_tec,vintage) ) ;

***

* **Possible extension:** Instead of assuming :math:`\beta_{n,t}` to be constant over time, one could include

* a simple (linear) projection of :math:`\beta_{n,t,y}` beyond the end of the model horizon. However, this would likely

* require to include the computation of dual variables endogenously, and thus a mixed-complementarity formulation of

* the model.

***

***

* Remaining installed capacity

* ----------------------------

* The model has to take into account that the technical lifetime of a technology may not coincide with the cumulative

* period duration. Therefore, the model introduces the parameter :math:`\text{remaining_capacity}_{n,t,y^V,y}`

* as a factor of remaining technical lifetime in the last period of operation divided by the duration of that period.

*

***

* Default 1: the full capacity is available over the entire period

remaining_capacity(node,tec,vintage,year_all)$map_tec_lifetime(node,tec,vintage,year_all) = 1;

* If the technical_lifetime ends within the respective period (year_all), the remaining capacity is computed as the

* ratio of the remaining lifetime as of the beginning of the period and the duration of the period.

remaining_capacity(node,tec,vintage,year_all)$(

map_tec_lifetime(node,tec,vintage,year_all)

AND (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all) < duration_period(year_all))

AND (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all) > 0)

) = (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all)) / duration_period(year_all);

* Same conditions as remaining_capacity, except inclusive of historic periods via map_tec_lifetime_extended.

* If MESSAGE_CAP_COMM is "0", map_tec_lifetime_extended is empty and all these are no-ops.

remaining_capacity_extended(node,tec,vintage,year_all)$map_tec_lifetime_extended(node,tec,vintage,year_all) = 1;

remaining_capacity_extended(node,tec,vintage,year_all)$(

map_tec_lifetime_extended(node,tec,vintage,year_all)

AND (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all) < duration_period(year_all))

AND (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all) > 0)

) = (technical_lifetime(node,tec,vintage) - duration_period_sum(vintage,year_all)) / duration_period(year_all);

* Mapping sets used in COMMODITY_BALANCE_AUX that are conditional on remaining_capacity_extended

map_cap_ret_hist_1(node,tec,vintage,y_prev,y_all)$(

first_period(y_all)

AND seq_period(y_prev,y_all)

AND inv_tec(tec)

AND map_tec_lifetime_extended(node,tec,vintage,y_prev)

AND NOT map_tec_lifetime_extended(node,tec,vintage,y_all)

AND (remaining_capacity_extended(node,tec,vintage,y_all) = 0)

) = yes;

map_cap_ret_hist_2(node,tec,vintage,y_prev,y_all)$(

first_period(y_all)

AND seq_period(y_prev,y_all)

AND inv_tec(tec)

AND map_tec_lifetime_extended(node,tec,vintage,y_prev)

AND (remaining_capacity_extended(node,tec,vintage,y_all) < 1)

AND (remaining_capacity_extended(node,tec,vintage,y_all) > 0)

) = yes;

* unassign the dynamic set 'year'

year(year_all) = no;